Note: Original script in “Ventilatory kinetics” does work but the run time for 161 files 23 minutes when downloaded to a desktop computer. Running it off of a flash drive (USB 3.0) on a laptop (Intel i5-9300H) took 38 minutes and 13 GB of RAM. The revised version of the script in the “Vent_kinetics_parallel_CPU_processing_V4” takes 7 minutes and about 1.5 GB of RAM when saved locally to a desktop computer for the same file set but runs your CPU at 100% for the whole time. Please be sure that your laptop fan is not covered when you run the script.

- Save all your data in .xlxs format

  - Use scripts to add HR and treadmill belt speed if necessary.

- Place your .xlxs data files in the “data” folder

- (ONLY IF NEEDED, as is the script will run all analyses as is) Edit “File_processing” to pull variables from the columns that you want to run

  - If you edit variables, make sure that you have python throw nan values (see the code for ‘Matched HR’ for reference)

  - Edit the results dictionary to include the variables that you want in your analysis

- Run the “Average-all-indices-initial-guesses-and-bounds.py” script to get the ball-park estimates for initial guesses for each measured parameter.

  - Open the output excel file and navigate to the baseline summary tabs for on- and off- kinetics

  - Then edit each model bounds in each of the scripts accordingly.

- (ONLY IF NEEDED) Edit “Batch_process” by commenting in/out the scripts that you want to run

  - This will then only run the scripts in the folder that you need and shorten analysis time

- Make sure all the files that python will be running the script on are closed.

  - Example 1: You cannot have the “Kinetics_summary.xlsx” file open when you run batch process.

  - Example #2: You also cannot have a participant treadmill data excel file open, it will throw an error and you will have to start the script over to include that participant.

- Run batch process by clicking the play button or pressing f5

- When batch_process is done, open the “Kinetics_summary.xlsx” file and view the results.

  - The file will have tabs that take the averages of the participant files from all files for a participant in _V1 and _V4 and a separate tab for the average of the second two tests (again divides the columns by visit, in this case _V1 and _V1)

    - If you have called the visit variables differently than the _V1_T1 format for example, you will need to change the script (or your saving convention) to match your naming convention or the variable to match the script.

Note: When trouble shooting, I put all data except for one excel file into the a folder called “Temp_data_storage” or “Trouble” to speed up the troubleshooting process.